BMW MINI NOV 2006 +  R55 CLUBMAN, COUPE, R56 MINI. R57 CABRIO

DISCONNECT BATTERY.  REMOVAL OF GRILLE & FITTING BRACKETS

Spot Light Kits should always be fit by qualified mechanics / vehicle electricians

: PULL OFF LOWER CHROME TRIM SLOWLY. PREY OUT THE FOUR TOP CLIPS LOCATED ON THE GRILL. AFTER THESE ARE REMOVED SQUEEZE EACH OF THE FOUR LOWER CLIPS AND GENTLY PULL THE GRILL STRAIGHT OUT.

: INSTALL THE BLACK LIGHT BRACKETS USING THE FACTORY BOLTS. WHEN FITTING THE BRACKET THE ‘N’ SHAPED PART MUST FACE UPWARDS. (BRACKET FITS TO BUMPER BAR NUT AND TORX SCREW FROM FRONT PANEL).

INSTRUCTIONS TO TRIM GRILL

: USE THE PICTURE GUIDE TO TRIM THE OPENING ON THE FRONT OF THE GRILL.

: COOPER S / ON BOTH THE LEFT AND RIGHT SIDE OF THE BACK OF THE GRILL YOU MUST TRIM THE FLAPS.

: COOPER / REMOVE THE LOWER CHROME TRIM AND CUT BACK THE BLACK  PLASTIC – THEN REINSTALL THE TRIM SO THAT THEY CLEAR THE BRACKETS.

Please note this next section is not always required. - Depending on the height of your bumper, you may have to trim the threaded tube on the bottom or the lamps by about .50 inch or 12mm (to clear the bumper top once they are installed).

The best way to trim the posts would be to install a threaded nut on the post then carefully cut the post with a hack saw or dremal tool. Remove the nut to restore the threads.

RECONNECT BATTERY – If lights fail check the earthing of your spot lamps, particularly if there are powder coated items in your set up.

FOR MINIS WITH CHROME SLATTED GRILLE

To feed the bracket through the chrome slatted grille you will need to cut away most of the black plastic material near the bottom slat (corners)  – you will see where this is as it lines up with the other trimmed grille allowing the brackets to pass through both.

For Wiring Instructions visit madautoparts.com – ‘how to fit spots’ .